Heater & Chiller Circulator

| Model Name |  | Heater chiller Circulator |
| --- | --- | --- |
| Model NO. |  | GDSZ-20/40EX |
| Temp accuracy(℃） |  | ±1 |
| Operate temp(℃) |  | -40～200℃ |
| Ambient temp(℃) |  | 5～35 |
| Best humidity |  | ≤60% |
| Power voltage |  | 1PH/220V/50HZ |
| Safety protection |  | Overvoltage, time delay, overcurrent, overheating, leakage, protector |
| Total power(kW/A) |  | 6.84/31.1 |
| Display |  | Digital control |
| Cooling capacity(kW) |  | 1.47 |
| Heating pow（kW) |  | 5 |
| Heat dissipating |  | Air-cold |
| Compressor | Item No. | MGu325Fxsf050×2 |
|  | HP | 2 |
|  | Type | Full sealed |
|  | Refrigerant | R404A |
| Cooling capacity(kW) | 100℃ | 4.62 |
|  | 30℃ | 4.3 |
|  | 0℃ | 3.16 |
|  | -5℃ | 2.62 |
|  | -10℃ | 2.23 |
|  | -40℃ | 0.37 |
| Cycle pump | Power(W) | 370 |
|  | Flow Rate (L/min) | 42 |
|  | Pressure(bar) | 2.2 |
| Cycle port size |  | DN15 |
| Communication port(optional) |  | RS485 |
| Heating storage tank |  | SUS304 |
| Heating transfer tube material |  | SUS304 |
| Expansion Tank (L) |  | 20 |
| Cycle medium |  | Silicon case 50cs below |
| Machine Size cm |  | About 66 W*80 L*95H |

Model No. GDSZ-20/40+200