Heater & Chiller Circulator

Model No.GDSZ series

- PLC type

- Explosive Proof  Type

Model No. GDSZ-100/40+200

Data sheet

| Model Name |  | Heater chiller |
| --- | --- | --- |
| Model NO. |  | GDSZ-100/20+200 |
| Temp accuracy(℃） |  | ±1 |
| Operate temp(℃) |  | -20～200℃ (300℃ is option,need extra cost) |
| Ambient temp(℃) |  | 5～35 |
| Best humidity |  | ≤60% |
| Power voltage |  | 3PH/380V/50HZ |
| Safety protection |  | Delay, over current, overheating, over temperature protection, over and under voltage, lack of phase |
| Total power(kW/A) |  | 19.2/31.1 |
| Display |  | PLC touch screen/digital control |
| Cooling capacity(kW) |  | 3.74 |
| Heating pow（kW) |  | 15 |
| Heat dissipating |  | Air-cold |
| Compressor | Item No. | YM86A1G-100 |
|  | HP | 5 |
|  | Type | Full sealed |
|  | Refrigerant | R404A |
| Cooling capacity(kW) | 100℃ | 20.61 |
|  | 30℃ | 20.61 |
|  | 0℃ | 13.82 |
|  | -5℃ | 11.48 |
|  | -10℃ | 9.43 |
|  | -15℃ | 7.66 |
|  | -20℃ | 2.89 |
| Cycle pump | Power(W) | 370 |
|  | Flow (L/min) | 42 |
|  | Lift（M） | 28 |
|  | Pressure(bar) | 22 |
| Cycle port size |  | DN20 |
| Communication port(optional) |  | RS485 |
| Heating storage tank |  | SUS304 |
| Heating transfer tube material |  | SUS304 |
| Expansion Tank (L) |  | 23 |
| Cycle medium |  | Silicon case 50cs below |
| Machine Weight |  | 160KG |

Model No. GDSZ-100/40+200

Data sheet

| Model Name |  | Heater chiller |
| --- | --- | --- |
| Model NO. |  | GDSZ-100/40+200 |
| Temp accuracy(℃） |  | ±1 |
| Ambient temp(℃) |  | -40～200℃ (300℃ is option,need extra cost) |
| Ambient temp(℃) |  | 5～35 |
| Best humidity |  | ≤60% |
| Power voltage |  | 3PH/380V/50HZ |
| Safety protection |  | Delay, over current, overheating, over temperature protection, over and under voltage, lack of phase |
| Total power(kW/A) |  | 21.5/34.5 |
| Display |  | PLC touch screen/digital control |
| Cooling capacity(kW) |  | 5.43 |
| Heating pow（kW) |  | 15 |
| Heat dissipating |  | Air-cold |
| Compressor | Item No. | YM125A1G-100 |
|  | HP | 7 |
|  | Type | Full sealed |
|  | Refrigerant | R404A |
|  | 0℃ | 29.64 |
|  | -5℃ | 17.69 |
|  | -10℃ | 14.74 |
|  | -30℃ | 6.6 |
|  | -40℃ | 4.64 |
| Cycle pump | Power(W) | 370 |
|  | Flow (L/min) | 42 |
|  | Lift（M） | 28 |
|  | Pressure(bar) | 22 |
| Cycle port size |  | DN20 |
| Communication port(optional) |  | RS485 |
| Heating storage tank |  | SUS304 |
| Heating transfer tube material |  | SUS304 |
| Expansion Tank (L) |  | 23 |
| Cycle medium |  | Silicon case 50cs below |
| Machine Weight |  | 170kg |