Heater & Chiller Circulator

Model No.GDSZ series

Model No. GDSZ-50/20EX

Data sheet

| Model Name |  | Heater chiller Circulator |
| --- | --- | --- |
| Model NO. |  | GDSZ-50/20EX |
| Temp accuracy(℃） |  | ±1 |
| Operate temp(℃) |  | -20～200℃ |
| Ambient temp(℃) |  | 5～35 |
| Best humidity |  | ≤60% |
| Power voltage |  | 3PH/380V/50HZ |
| Safety protection |  | Overvoltage, time delay, overcurrent, overheating, leakage, protector |
| Total power(kW/A) |  | 10.12/16.96 |
| Display |  | PLC touch screen/digital control |
| Cooling capacity(kW) |  | 2.25 |
| Heating pow（kW) |  | 7.5 |
| Heat dissipating |  | Air-cold |
| Compressor | Item No. | YM49A1G-100 |
|  | HP | 3 |
|  | Type | Full sealed |
|  | Refrigerant | R404A |
| Cooling capacity(kW) | 100℃ | 11.62 |
|  | 30℃ | 11.62 |
|  | 0℃ | 8.28 |
|  | -5℃ | 6.93 |
|  | -10℃ | 5.78 |
|  | -15℃ | 4.78 |
|  | -20℃ | 3.94 |
| Cycle pump | Power(W) | 370 |
|  | Flow Rate (L/min) | 22 |
|  | Pressure(bar) | 2.2 |
| Cycle port size |  | DN15 |
| Communication port(optional) |  | RS485 |
| Heating storage tank |  | SUS304 |
| Heating transfer tube material |  | SUS304 |
| Expansion Tank (L) |  | 20 |
| Cycle medium |  | Silicon case 50cs below |
| Machine Size cm |  | About 66 W*65 L*150H |